Activity Title: Comprehensive Incident Response and Threat Analysis for SCADA Security

Objective: The CSIRT team will conduct a detailed analysis of the network devices and SCADA components involved in the gas distribution control system at Allander to identify potential security threats and anomalies. This analysis will generate security events data, system logs, network packet captures, malware samples for reverse engineering, performance data, and threat actor profiles.

Duration: 6 weeks

Step 1: Security Events Data Generation and Analysis

- Task: Generate and analyze security events from network devices such as firewalls, intrusion detection systems (IDS), intrusion prevention systems (IPS), and Westermo Ethernet extenders.

- Actions:

  - Configure the SIEM system to aggregate logs and security events from all critical devices (RTUs, PLCs, Westermo extenders, firewalls, IDS/IPS).

  - Simulate cyber incidents (such as port scans, brute-force attacks, and unauthorized access attempts) using penetration testing tools to generate security events.

  - Analyze the logs in the SIEM dashboard to identify key events (e.g., login failures, anomaly detection alerts) and correlate them with potential attack vectors.

- Expected Output: A report detailing the types of security events detected, including false positives, successful correlations of incidents, and any undetected vulnerabilities.

Step 2: System Logs Collection and Analysis

- Task: Collect and review system logs from  SCADA components (e.g., RTUs, PLCs, firewalls, servers) to identify any misconfigurations, unauthorized changes, or suspicious activities.

- Actions:

  - Collect logs from all network devices, SCADA systems, and operator workstations.

  - Analyze logs for anomalies such as unexpected shutdowns, configuration changes, failed login attempts, or irregular network communication between SCADA components.

  - Set up specific alerts for privileged account usage and system modification attempts in the SIEM system.

- Expected Output: A detailed log analysis report identifying potential security risks, including unauthorized access attempts, configuration changes, and system irregularities.

Step 3: Network Packet Capture and Analysis

- Task: Capture network traffic data from critical network paths between SCADA components and analyze it for anomalies such as unusual traffic spikes, data exfiltration attempts, or suspicious protocol usage.

- Actions:

  - Use packet capture tools (e.g., Wireshark) on the network, especially on segments involving the Westermo Ethernet extenders, firewalls, and SCADA communication lines.

  - Look for signs of malicious activity, including:

    - Traffic to/from unusual IP addresses.

    - Unencrypted sensitive data transmission.

    - Use of non-standard or SCADA-specific protocols (Modbus, DNP3, etc.).

  - Identify potential reconnaissance or lateral movement attempts by threat actors.

- Expected Output: A packet capture analysis report summarizing any unusual traffic patterns, identifying potential network-based attacks or misconfigurations, and detailing encrypted or sensitive data movement.

Step 4: Malware Reverse Engineering

- Task: If any malware is discovered during network monitoring or endpoint security analysis, the CSIRT team will conduct reverse engineering to understand its behavior and potential impact on the SCADA system.

- Actions:

  - Collect any suspicious files or executables found on SCADA components or operator workstations.

  - Perform static and dynamic analysis of the malware in a controlled sandbox environment (using tools like IDA Free, Ghidra, or ANY.RUN Sandbox).

  - Identify key malware characteristics, such as file signatures, communication methods, payloads, and persistence mechanisms.

- Expected Output: A reverse engineering report that includes malware behavior, attack mechanisms, Indicators of Compromise (IoCs), and recommendations for detection and prevention.

Step 5: Hardware/Virtual Resource Performance Monitoring

- Task: Monitor and analyze hardware/virtual resources across the SCADA infrastructure to detect performance degradation or unusual resource utilization that could indicate an ongoing attack.

- Actions:

  - Set up performance monitoring tools (e.g., Nagios, Zabbix) on key devices such as SCADA servers, operator terminals, and network devices (routers, switches, etc.).

  - Analyze CPU, memory, disk, and network interface utilization for spikes or anomalies that coincide with potential security incidents.

  - Investigate any system slowdowns or performance issues that may result from malware activity, DoS attacks, or misconfigurations.

- Expected Output: A performance analysis report indicating any resource anomalies and correlating them with potential security threats or operational inefficiencies.

Step 6: Threat Actor Profile Development

- Task: Based on all collected data (events, logs, network traffic, and malware analysis), develop profiles of potential threat actors targeting the SCADA system.

- Actions:

  - Correlate attack patterns, tools, techniques, and procedures (TTPs) with known threat actor profiles using threat intelligence platforms (e.g., MITRE ATT&CK framework, MISP).

  - Determine the most likely threat actor types, such as hacktivists, nation-state actors, or financially motivated cybercriminals.

  - Profile the types of vulnerabilities and attack vectors that threat actors could exploit in the SCADA environment.

- Expected Output: A comprehensive threat actor profile, including possible motivations, attack methods, and recommended defensive strategies to mitigate future attacks.

Final Deliverables:

At the end of the activity, the CSIRT team will produce the following:

- Security events data report.

- Detailed system log analysis.

- Network packet capture analysis with identified anomalies.

- Malware reverse engineering report (if applicable).

- Hardware/virtual resource performance report.

- Threat actor profile report.

These reports will inform management and security teams of the vulnerabilities, threats, and recommended mitigation measures to strengthen the SCADA system's cybersecurity posture.

Post-Activity Reflection:

After completing the activity, the CSIRT team will hold a debriefing session to:

- Review key findings.

- Identify areas of improvement in the current security setup.

- Refine detection rules, signatures, and alerts in the SIEM system for future use.